What's in a Map?

Part 1: Basic Requirements

Maps are the primary tools by which spatial relationships are depicted. Maps therefore become important documents. There are several key elements that should be included each time a map is created in order to aid the viewer in understanding the communications of that map. The recording of spatial data on maps is known as cartography.

### Required Elements

1. Data Frame

The data frame is the portion of the map that displays the data layers. This section is the most important and central focus of the map document. In the example displayed at the end of this article, the data frame contains fire history for the community of Topanga.

2. Legend

The legend serves as the decoder for the symbology in the data frame. Therefore, it is also commonly known as the key. Descriptions detailing any color schemata, symbology or categorization is explained here. In the legend below, the fire history schemata has been categorized with a graduating color scheme. The legend details which colors refer to which years. Without the legend, the color scheme on the map would make no sense to the viewer. The legend tells the viewer that the lighter the color, the longer the last recorded date of fire has been.

3. Title

The title is important because it instantly gives the viewer a succinct description of the subject matter of the map. The title "Fire History in Topanga, California" quickly tells the viewer the subject matter and location of the data.

4. North Arrow

The purpose of the north arrow is for orientation. This allows the viewer to determine the direction of the map as it relates to due north. Most maps tend to be oriented so that due north faces the top of the page. There are exceptions to this and having the north arrow allows the viewer to know which direction the data is oriented.

5. Scale

The scale explains the relationship of the data frame extent to the real world. The description is a ratio. This can be shown either as a unit to unit or as one measurement to another measurement. Therefore a scale showing a 1:10,000 scale means that every one paper map unit represents 10,000 real world units. For example 1:10,000 in inches means that a measurement of one inch on the map equals 10,000 inches in real life. The second method of depicting scale is a comparison with different unit types. For example, 1":100' means that every inch measure on the paper map represents 100 feet in the real world. This ratio is the same as 1:1200 (1 foot = 12 inches). In addition to text representation as described above, the ratio can be shown graphically in the form of a scale bar. Maps that are not to scale tend have have a "N.T.S" notation which stands for "Not to scale."

6. Citation

The citation portion of a map constitutes the metadata of the map. This is the area where explanatory data about the data sources and currency, projection information and any caveats are placed. In the example below, the citation tells the source and date of the data. Citations help the viewer determine the use of the map for their own purposes.

### Optional Elements

Further elements, though not required, can be placed on the map to further aid the viewer.

7. Border

Placing a border on the map helps to bind all the map elements together. In addition, internal borders serve to categorize the map elements. A border is also referred to as a neatline.

8. Overview map

Especially when zoomed in on a region, an overview map is helpful to orient the viewer not familiar with the area of interest. If you don't live in the Los Angeles area, you probably aren't familiar with the community of Topanga. Adding an overview map gives the viewer a visual concept of Topanga's location within California.

9. Graticules and Indexes

Graticules are grids that overlay on the data frame. They can be lines of latitude and longitude or an index grid. They help orient the viewer on the map. An index is an arbitrary imposed set of cells over the map that helps guide the viewer to a specific portion of the map from an index. This is most commonly found in travel maps and atlases.

Many other elements can be added to a map such as text, charts and images. This article reviewed the most common and basic elements likely to be found on a paper map.