أتبع الخطوات بدقة ولا تنسى فضل صاحب النص البرمجي، ولا تنسانا من دعائك. ناقل النص  (د. علي الغامدي)

This ArcMap field calculator script will calculate the bearing (North Azimuth format) between two sets of x y coordinates in an attibute table. This script was designed for plane coordinates (stateplane, UTM) and not geographic coordiantes, and I don't think it will work correctly for data in DD. I wrote this script beacuse I wanted to know what direction I had moved a point from it's original location. Remember, this is for ArcInfo, it might work for ArcView

Procedure: 
1. Calculate the X,Y coordinates for a point layer (Xtools or whatever). 
2. Rename the X,Y fields to "X_ORIG" and "Y_ORIG". 
3. Edit the point locations (move the points). 
4. Recalculate the X,Y coordinates for a point layer. 
5. Make a new field in the attribute table that is of type Double with at least 3 spaces for integer values (i.e. 259.12 degrees). Precision of 10, scale of 7 for example. 
6. Copy and paste the script into the Field Calculator's "Pre-Logic VBA Script Code" window. See the comments in the script file for help. 
7. Enter "dBearing" in the lower window (the variable you want returned to the new field). 
8. Click OK to run.

'Be sure to check the "Advanced" checkbox in the Field Calculator first.

'Enter the following code into the "Pre-Logic VBA Script Code" window

'and then enter "dBearing" into the "FIELDNAME =" window.

'Note that a return value of 999.999 means the point did not move.

'Also replace the field names "[X_ORIG]", "[Y_ORIG]", "[X]", and "[Y]"

'with your field names ("_ORIG" being the original coordinate). This script

'may be useful if you want to know what direction a point has moved after you

'edit it. Ofcourse you must first derive the original x,Y coordinates, rename

'the fields, and then calc new X,Y coords after you edit the point locations.

Here is the code:

Dim Dx as Double

Dim Dy as Double

Dim Pi as Double

Dim dBearing as Double

Pi = 4 * Atn (1)

Dx = ([X_ORIG] - [X])

Dy = ([Y_ORIG] - [Y])

If Dx < 0 Then

dBearing = 90- (180/Pi) * Atn(Dy/Dx)

End If

If Dx > 0 Then

dBearing = 270 - (180/Pi) * Atn(Dy/Dx)

EndIf

If Dx = 0 Then

If Dy < 0 Then

dBearing = 0

End If

If Dy > 0 Then

dBearing = 180

End If

If Dy = 0 Then

dBearing = 999.999

End If

End If

'Don't forget to put "dBearing" into the "FIELDNAME =" window.

'Thanks to Doctor Rick from http://mathforum.org/library/drmath/view/62034.html

'for the equations. Also, here is the equation for the distance between points:

'DIST = Sqr(([X]-[X_ORIG])^2+([Y]-[Y_ORIG])^2) - that one's easy!

'Chris Snyder - 11/18/2004

'chris.snyder@wadnr.gov